Material Safety Data Sheet

1.CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

PRODUCT NAME: PU 2-COMPONENT DRY LAMINATION PASTE MAIN RESIN

TRADE NAME : AEDLC

Manufacturer/supplier: Company Name: NAN YA PLASTICS CORPORATION Address: No.6 KONG YEH 1st ROAD 832 LIN YUAN INDUSTRISL ZONE, KAOHSIUNG, TAIWAN R.O.C. Emergency Telephone / Fax No.: 886-7-6419911-683 / 886-7-6432546

2.Composition/information on ingredients

| MIX SUBSTANCE |  |  |  |
| --- | --- | --- | --- |
| Component（s） | Catalogue of Hazards | C. A. S. No. | Percentage |
| OH-terminated urethane prepolymer |  |  | 64.0%~74.0% |
| ETHYL ACETATE | 3 flammable | 141-78-6 | 26.0%~36.0% |

3. Hazardous identification

| Warning： ○Irritation to the skin, eyes and respiratory tract. ○May be absorbed through the skin | Protective Measures： ○Avoid contact with the skin, eyes and wear proper skin protection |
| --- | --- |
| Critical Hazards: Flammable liquid and vapor. Vapor may ignite explosively. |  |
| Class / Division: 3 FLAMMABLE |  |

4. FIRST AID MEASURES

Inhalation: remove person to fresh air. if breathing is difficult , provide oxygen and seek medical attention

Skin Contact: Take off the clothing and wash with water and soap.

Eye contact: Immediately flush eyes with a large amount of water for at least 15 minutes, and get medical attention .

Ingestion: Do not induce vomiting. Transfer to medical facility for gastric lavage.

Suck or taking will cause toxicity.

5. FIRE FIGHTING MeasureS

EXTINGUISHING MEDIA/ FIRE FIGHTING PROCEDURES

Use the following extinguishing media when fighting fires involving this material: water fog, foam, dry chemical power, or carbon dioxide.

Special risk: Explosive when organic vapor heated.

Special extinction procedures: 1.Fire involving should be fought upwind and wear a suitable set of protective clothing. 2.Water spray may be used to cool fire exposed containers, but the spray may not extinguish fire.

Firefighters should wear a self-contained breathing apparatus and breathe the vapor.

6. Accidental release measures

# release response

Remove unnecessary personnel from the area. Avoid inhalation of the vapors and contact with the skin. Avoid leakage to the drainage system. Contain with an absorbent material, such as paper, cloth towel or sand, and transfer to a suitable container for disposal. Wash down the spill area by water.

PERSONAL PROTECTIVE EQUIPMENT

Wear chemical splash goggles, impervious body covering and rubber gloves. Wear an air-purifying respirator equipped with cartridges for protection against organic vapors.

containment and cleaning:

1.Promply remove ignition sources, both actual and potential.. 2.Small spills may be absorbed with dry earth or sand, or wiped off with rag, and placed into a chemical waste containers. 3.In case of large spills, dike with earth or sand to guide the flow to a safe area for collection and recovery 4.To burned in the incinerator. 5.To bury the solid wastes in the qualified waste yard. 6.To freight with the qualified transportation.

7. Handing and Storage

Handing: 1.When using do not wear contact lens. Always wear protect equipments. 2.After handling wash hands thoroughly and change the contaminated clothing. 3.Pay attention to the management of empty containers as well. 4.Prevent generation of static electricity. Clothing and work shoes must be electroconductive. 5.Ground and bond containers when transferring material. 6.Wear rubber gloves, protective mask

Storage: 1.Store in a ventilated area. 2.The product content low flash point evaporated solvent, when it is applied, fire is prohibited. 3.Electrical equipment must be of explosion-proof construction and ground. 4.Keep away from ignition sources, high temperatures as boiler, and oxidizing materials.

8. Exposure ControlS/PERSONAL PROTECTION

occupational Exposure limits

# Ethyl acetate: ACGIH(1997) TLV-TWA 400ppm # OSHA(1997) PEL-TWA 400ppm

## eNGINEERING Control MEASURES

Use local exhaust ventilation if material is heated or missing.

Personal Protection Equipment:

Respiratory Protection: NIOSH approved one

Hand Protection: Impermeable gloves

Eye Protection: Use safety chemical goggles

Skin & Body Protection: Use Anti-splash Clothing

Industrial Hygiene: 1. To install emergency eye wash fountains and shower at the potential exposed zone. 2. Thoroughly wash the polluted clothing before next use. 3. Should thoroughly wash hands with soap and water before eating, using tobacco products, using toilet facilities. 4. Always take shower with clean water and soap after work.

9. Physical and Chemical Properties

| Form: liquid | Color: pale yellow clear liquid | Odor: fragrant odour |
| --- | --- | --- |
| P.H. Value: | Boiling Range: 77℃ | Decomposition Temperature: |
| Flash Point: -4.4℃ for ethyl acetate | Explosion Limits: 2~11.5% by vol for ethyl acetate | Solubility in water: insoluble |
| Specific Gravity: 0.9 | Vapor Pressure: 3 .04for ethyl acetate | Melting point: |

10. Stability and Reactivity

Chemical Stability: Stable

Possible hazardous reaction under extreme conditions: Will not occur Conditions to avoid : heat Decomposition: Smog, CO, CO2, NOx Incompatibility with other materials: none

11. ToxicOLOGICAL Information

Ingestion: low or no toxic as for acute toxicity. LD50(rat)=5,620mg/kg for ethyl acetate Eye : liquid irritates the eyes and the vapour of ethyl acetate irritate eye tissue. Skin : prolonged or repeated skin contact may cause dermatitis. Inhalation: the vapour of ethyl acetate irritates respiratory tract and has narcotic effect. High concentration of ethyl acetate cause headache, drowsiness and stupor. LC50(rat)=16,000ppm/6H for ethyl acetate.

12. ECOLOGICAL INFORMATION

BIODEGRADATION: no information, but estimated to be not appreciably toxic.

ACCUMULATIVENESS: N/A

13. DISPOSAL CONSIDERATION

Comply with all national or local recommendations and regulations.

14. TRANSPORT INFORMATION

| International Transport Regulation: this product is dangerous goods in IMDG(sea transport) status and ICAO(air transport) status. | UN No. : 1133 |
| --- | --- |
| Local Transport Regulation: N/A | Hazard class: 3.2 |
| Special Transportation & Consideration: Avoid transported with incompatible goods. |  |

15. REGULATORY INFORMATION

Comply with each country's regulation.

16. Other Information

The data here is based on our current knowledge and experience. The data can be revised by new information. The purpose of this Safety Data Sheet is to describe the products from the point of view of safety requirements. Therefore it should not be construed as guaranteeing specific properties.

Manufacturer: NAN YA PLASTICS CORPORATION

No.6, Kong Yeh 1st Rd., 832 Lin Yuan Industrial Zone, Kaohsiung, Taiwan (R.O.C.)

Tel: 886-7-6419911 EXT 650 Fax: 886-7-6432546

Cheng Jen-liang (Factory Director)

DATE OF MSDS : 2014/12/17